T2Sem-DRL: Research Idea and Results

Jiashang Shen | Wenzhou Kean University

1. Background and Problem

• Scenario: multi-UAV + HAP hierarchical MEC with semantic compression ratio μ; each task must satisfy delay and similarity SLAs.

• Three SLA tiers: Critical (≤100 ms, sim≥0.92), Interactive (≤300 ms, sim≥0.85), Best-effort (≤800 ms, sim≥0.75).

• Bottlenecks: (i) structurally infeasible offload actions in continuous spaces (pure DRL: 59.9% SLA); (ii) tier starvation under single-metric optimization (greedy: 91.6% critical but 49.9% interactive); (iii) UAV trajectory energy with diminishing SLA return once routing is resolved by feasibility projection.

2. Technical Approach (T2Sem-DRL v4.3)

• Feasibility projection F: analytic semantic/channel/queue models clip actions before execution; all baselines share F for fair comparison.

• Hierarchical semantic MEC model: adaptive μ, three-way offloading (local/UAV/HAP), contention-aware queueing.

• Per-tier CMDP (PPO-Lagrangian): independent λ_κ for tier diagnostics, not aggregate-SLA improvement (single-λ 92.80% vs. full 92.17%).

• Masked PPO + optional two-timescale DDPG: PPO refines within F (+6.2 pp); DDPG is an optional SLA–energy mode (93.99% @ 7.9 kJ vs. 88.85% @ 4.1 kJ).

3. Experimental Setup

• Training: 300 episodes × 3 seeds {42,123,456}; evaluation: 30 deterministic episodes per seed.

• Scale: 8 UEs / 16 terminals / 3 UAVs / 2 HAPs; WSL Ubuntu CPU.

• Baselines: Random, Greedy, TJCCT-Lite, Local-only, Independent PPO, Single-timescale.

• Ablations: mask, semantic, per-tier λ, CMDP, single-ts, fixed-μ, and more (10 variants).

• Stress test: mobility-critical low-HAP-capacity scenario (Table VI).

4. Main Results

• Table I — T2Sem-DRL: 93.99±1.74% SLA, 0.100 s delay; Greedy 87.77%, TJCCT 86.64%.

• Table II — T2Sem-DRL balances tiers: 76.0% critical, 99.2% interactive, 100.0% best-effort; greedy sacrifices interactive (49.9%).

• Table IV/V — Ablation hierarchy: semantic −46.0 pp, mask −31.8 pp, RL +6.2 pp over masked greedy.

• Table I-b — Energy as operating points; no universal energy-dominance claim.

• Table VI — Mobility-critical: two-timescale 16.4% vs. single-ts 1.7%; reported as limitation.

5. Current Status

• Full revision + ablation + mobility stress experiments completed.

• 8 publication figures (blue palette + Times New Roman; Fig.8 high-contrast exception).

• IEEE Word manuscript with native OMML equations; references and figure numbering verified.

• Optional next steps: semantic MAPPO baseline, statistical tests, submission formatting.

6. Suggested Narrative for Advisor Meeting

• Problem: infeasible actions and tier starvation dominate, not missing DRL architecture.

• Method: F + semantic model as system-level interface; RL refines inside the shell.

• Evidence: ablation quantifies honest hierarchy (semantic > mask > RL).

• Honesty: do not overclaim per-tier λ or DDPG; report energy as operating points; acknowledge critical-tier and mobility-critical limits.

• Deliverables: complete manuscript + reproducible code (t2sem-mec-v2/).