T2Sem-DRL: Per-Tier Constrained Semantic Offloading with Model-Informed Feasibility in Multi-UAV Hierarchical MEC

Jiashang Shen

Wenzhou Kean University, Wenzhou, China

shenjias@kean.edu

Abstract

Hierarchical UAV–high-altitude platform (HAP) mobile edge computing (MEC) with semantic communication must satisfy heterogeneous per-tier SLAs (delay and similarity) under stochastic air-ground channels. We argue that the dominant bottleneck is not a missing DRL architecture but structurally infeasible offload decisions in continuous action spaces and tier starvation under single-metric optimization.

We propose a model-informed constrained semantic offloading interface comprising: (1) a feasibility projection F that clips (μ, o) using analytic semantic, channel, and queueing models—shared by all policies as a system-level action interface; (2) per-tier constrained CMDP (PPO-Lagrangian) with independent dual variables λ_κ for tier-level diagnostics; (3) optional masked PPO to refine routing within F. Final v4.3 ablations quantify an honest contribution hierarchy on overall SLA: semantic modeling +46.0 pp, feasibility mask +31.8 pp, and masked RL over masked greedy +6.2 pp. Per-tier λ_κ does not improve aggregate SLA in the final run (92.17% full vs. 92.80% single-λ), so we present it as a tier-allocation mechanism rather than a primary accuracy lever. Two-timescale DDPG improves default SLA over the single-timescale variant (93.99% vs. 88.85%) but roughly doubles energy; it is retained as an optional operating mode, not the central contribution.

Over 300 episodes and three seeds, the final full model achieves 93.99±1.74% overall SLA and 0.100 s mean delay, compared with masked greedy at 87.77±0.07% and TJCCT-Lite at 86.64±0.07%. We do not claim energy superiority over static heuristics under the default training checkpoint; energy is reported separately with operating points and single-timescale variants.

Index Terms— UAV, mobile edge computing, semantic communication, constrained Markov decision process, service-level agreement, task offloading, feasibility region, proximal policy optimization.

I. INTRODUCTION

A. Motivation and Application Context

Mobile edge computing (MEC) offloads computation-intensive workloads from user equipments (UEs) to edge servers, reducing end-to-end (E2E) latency relative to remote cloud execution [17]. Unmanned aerial vehicles (UAVs) extend MEC coverage in disaster response, rural broadband, and temporary event networks by acting as aerial relays with line-of-sight (LoS) dominance [1], [13]. High-altitude platforms (HAPs) further provide wide-area compute at altitudes of tens of kilometers, forming a hierarchical aerial MEC architecture: local UE arrow UAV arrow HAP [2], [8].

Concurrently, semantic communication (SemCom) transmits task-relevant feature representations rather than raw bits, controlled by a semantic compression ratio μ ∈ (0,1] [11], [20]. Lower μ reduces radio load but increases semantic extraction delay and risks violating minimum similarity requirements. Modern applications impose heterogeneous SLAs: emergency sensing (critical tier) demands sub-100 ms latency and high similarity; interactive augmented reality (interactive tier) tolerates hundreds of milliseconds; best-effort analytics relax both constraints.

Problem statement. We study semantic-SLA-aware task offloading in a multi-UAV, multi-user, multi-terminal hierarchical semantic MEC system: at each time slot, every terminal decides (i) offload target among local CPU, a UAV, or a HAP, (ii) semantic ratio μ, and (iii) bandwidth/CPU resource shares; at each long frame, UAVs update 3D trajectories. The objective is to maximize tier-aware SLA satisfaction while limiting UAV propulsion energy.

B. Limitations of Prior Methods

Convex and heuristic optimization. Optimization-based MEC resource allocation methods [16], [17] provide useful structure but often assume known statistics and do not scale cleanly to high-dimensional stochastic dynamics with semantic tiers. TJCCT [5] decouples UAV trajectory (long timescale) from resource allocation (short timescale) via matching and convex optimization rather than end-to-end learning.

Single-metric DRL offloading. Multi-UAV DRL frameworks [1], [2] optimize delay, energy, or priority-weighted utility but not explicit multi-tier semantic SLAs. Semantic-aware MAPPO offloading [4], [5] lacks hierarchical HAP cooperation and two-timescale aerial control.

Single-constraint Lagrangian RL. Constrained MDP (CMDP) methods [14], [15] enforce expected-cost constraints via dual ascent. A single global Lagrange multiplier cannot simultaneously guarantee critical, interactive, and best-effort tiers when their feasible regions conflict: our per-tier evaluation (Table II, Fig. 4) shows greedy policies reach 91.6% critical SLA but only 49.9% interactive SLA.

Continuous offload actions without feasibility. Mapping a continuous offload preference to local/UAV/HAP targets yields many structurally infeasible decisions (e.g., local compute cannot meet critical 100 ms deadline under nominal task size). Without model guidance, DRL wastes samples on actions that fail regardless of learning (pure DRL ablation: 59.9% SLA, Appendix B).

C. Technical Challenges

This work addresses three coupled challenges:

• Tier-heterogeneous QoS (C1). Aggregate SLA optimization masks per-tier starvation. A policy can maximize overall satisfaction while failing an entire tier class.

• Semantic–offloading infeasibility (C2). Continuous action spaces do not encode analytic delay/similarity limits. Under stochastic air-ground channels and ± 20\% task-size jitter, optimistic routing misses deadlines.

• Energy–SLA coupling via UAV mobility (C3). Long-timescale trajectory learning consumes propulsion energy [1], [13]. When routing is resolved by feasibility projection, excessive UAV movement yields high energy without SLA gain.

D. Proposed Framework and Positioning

Primary contribution is not a new two-timescale DRL recipe but a model-informed per-tier semantic SLA interface:

• Feasibility projection F (dominant lever). Before execution, project actions onto analytic delay/similarity limits (Section IV-B). Removing F costs 31.8 pp in the final ablation (60.39% vs. 92.17% full). All baselines—including greedy, TJCCT, and random—share F for fair comparison.

• Semantic–offloading model (dominant lever). Adaptive μ and remote load modeling contribute +46.0 pp over the no-semantic ablation (46.17% vs. 92.17%). Without semantics, even RL cannot recover tier-critical feasibility.

• Masked PPO (secondary lever). Within F, learning improves masked greedy by +6.2 pp overall (93.99% vs. 87.77%) and lowers mean delay from 0.122 s to 0.100 s—meaningful but smaller than F and semantics.

• Per-tier CMDP (tier-allocation lever). Independent λ_κ does not improve aggregate SLA in the final ablation (single-λ 92.80% vs. full 92.17%) but exposes tier-level trade-offs that are hidden by a single global constraint (Table II).

• Two-timescale DDPG (optional). At default UE density and HAP capacity, single-timescale control reaches 88.85% SLA at 4.1 kJ, while the full two-timescale model reaches 93.99% at 7.9 kJ. DDPG is retained as an optional SLA-energy operating mode rather than the core scientific contribution.

Honest hierarchy (overall SLA, ablation scale): semantic (+46.0 pp) > mask (+31.8 pp) > RL over masked greedy (+6.2 pp), while per-tier λ_κ mainly changes tier allocation.

Fig. 1. T2Sem-DRL v4 architecture and model-informed feasibility interface.

E. Contributions

The main contributions are:

• Model-informed feasibility region F as a system-level action interface with delay margin α, expected-channel estimation, and similarity headroom δ_s (Section IV-B; +31.8 pp ablation gain).

• Hierarchical semantic MEC model coupling μ, three-way offloading, contention, and three-tier SLAs (+46.0 pp when semantics is removed).

• Per-tier constrained CMDP formulation with independent dual ascent; emphasizes tier diagnostics and allocation over aggregate-SLA improvement.

• Masked PPO policy learner atop F (+6.2 pp over masked greedy; not claimed as the primary scientific advance).

• Unified evaluation protocol: masked baselines, cumulative ablation (Table V), energy operating points (Table I-b), and explicit non-claims on default trajectory energy.

F. Paper Organization

Section II reviews related work. Section III presents the system model. Section IV formulates the CMDP and feasibility region. Section V describes the T2Sem-DRL algorithm and its components. Section VI lists implementation details. Section VII reports experiments. Section VIII concludes.

II. RELATED WORK

A. UAV-Assisted MEC and Trajectory Optimization

UAV-assisted MEC jointly optimizes communication, computation, and flight [1], [12], [13]. DRL methods learn trajectory and offloading adaptively [1], [2], [12], [13] but typically optimize scalar delay/energy objectives. DTOTO [2] considers hierarchical UAV–HAP MEC without semantic compression or tiered SLAs. Our work adds SemCom and per-tier CMDP on top of hierarchical offloading.

B. Semantic Communication and Semantic-Aware MEC

SemCom replaces bit-level fidelity with task-level similarity [9]–[11], [20]. Resource allocation with semantic extraction appears in [3]; online multi-task semantic offloading uses MAPPO in [4]. UAV semantic MEC studies [11], [21], [22] do not address HAP hierarchy or constrained RL. We integrate semantic μ control with three-way offloading and analytic feasibility.

C. Two-Timescale Edge Resource Management

Two-timescale decomposition reduces action dimensionality: slow variables (trajectory, caching) vs. fast variables (offloading, bandwidth) [5]–[8]. TJCCT [5] uses optimization rather than DRL for the fast scale. Related two-timescale MEC/caching studies [6], [7] show the value of temporal decomposition, while HAPS-assisted MEC [8] motivates the aerial hierarchy. We adopt PPO+DDPG with frame-aggregated rewards and energy regularization.

D. Constrained Reinforcement Learning

CMDPs enforce safety or QoS constraints via Lagrangian methods [14], [15]. Multi-constraint problems require multiple dual variables, one per constraint. Prior aerial offloading RL uses penalty weights, not per-tier dual ascent. We extend PPO-Lagrangian to per-tier bounded violation costs with independent λ_κ updates.

E. Positioning Summary

| Work | UAV | HAP | Sem. μ | SLA tiers | Per-tier dual | Feas. mask | Two-scale | DRL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SemCom survey [9] | — | — | ✓ | — | — | — | — | — |
| Hao TMC'24 [1] | ✓ | ✗ | ✗ | priority | ✗ | ✗ | ✗ | ✓ |
| DTOTO [2] | ✓ | ✓ | ✗ | delay | ✗ | ✗ | ✗ | ✓ |
| Sem MAPPO [4] | ✗ | ✗ | ✓ | ✗ | ✗ | ✗ | ✗ | ✓ |
| UAV Sem [11], [21] | ✓ | ✗ | ✓ | ✗ | ✗ | ✗ | ✗ | partial |
| TJCCT [5] | ✓ | ✗ | ✗ | ✗ | ✗ | ✗ | ✓ | partial |
| Two-scale caching/MEC [6], [7] | ✗ | ✗ | ✗ | QoS | ✗ | ✗ | ✓ | ✓ |
| T2Sem-DRL v4 | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |

Gap. No prior work jointly provides hierarchical UAV–HAP semantic offloading, per-tier CMDP, and model-informed feasibility projection in a unified two-timescale DRL framework.

III. SYSTEM MODEL

A. Network and Workload

Consider a geographic area of size L × L m^2. Index set of UEs N=1,…,N; each UE n has a set of terminals T_n with |T_n|=T_u and N_t = NT_u terminals total. M UAVs M=1,…,M fly at altitude H_u with positions q_m(t)=[x_m,y_m,z_m]^T. K HAPs K=1,…,K are fixed at q_k^hap with altitude H_h ≫ H_u.

Time is slotted with duration Δ. A long frame t comprises K_s slots k=1,…,K_s. UAV trajectory updates occur at frame boundaries; per-slot offloading decisions occur every slot.

Each terminal i generates tasks according to a Bernoulli arrival with rate λ_i per slot. Task T_i is characterized by:

• Raw data size D_i (bits), CPU cycles C_i (cycles);

• SLA tier κ(i) ∈ K_SLA=critical,interactive,best\_effort;

• Requirements (T_κ^max, S_κ^min).

Default tier parameters (simulation): critical (0.1 s, 0.92); interactive (0.3 s, 0.85); best-effort (0.8 s, 0.75).

B. Semantic Communication Model

Terminal i selects semantic compression ratio μ_i ∈ [μ_min, 1].

Transmitted data volume:

Ditx=μiDi.	(1)

Semantic extraction delay at local CPU frequency f_i (Hz) with coefficient c_sem:

Tisem=csem1μi−1Difi.	(2)

Equation (2) implies that as μ_i arrow 0, extraction time grows unbounded—modeling the cost of aggressive compression.

Achieved semantic similarity [implementation: similarity()]:

S(μi)=Smax1−exp(−λaccμi),	(3)

where S_max is the maximum similarity and λ_acc controls accuracy gain rate.

Minimum μ for tier κ (similarity feasibility):

μκsim=−1λaccln1−Sκmin+δsSmax,	(4)

with similarity headroom δ_s ≥ 0 (default 0.05).

Remote computation load when offloading:

Ciremote=μiCi.	(5)

C. Air-Ground Channel Model

3-D distance between terminal position p_i and aerial node q:

di,𝐪=‖𝐩i−𝐪‖2.	(6)

Elevation angle (degrees):

θi,𝐪=arctanz𝐪−zi‖𝐩ixy−𝐪xy‖2·180π.	(7)

LoS probability (ITU-style) [19]:

PLoS(θ)=11+aexp(−b(θ−a)).	(8)

Path loss (dB) for distance d and carrier frequency f_c:

PLLoS(d)=20log10d+20log10fc−147.55,	(9)

PLNLoS(d)=PLLoS(d)+20.	(10)

During simulation, LoS/NLoS is sampled; for feasibility estimation we use expected gain:

gi,𝐪=PLoS(θ) 10−PLLoS(d)/10+(1−PLoS(θ)) 10−PLNLoS(d)/10.	(11)

Alternatives: optimistic LoS-only (g = 10^-PL_LoS/10) or pessimistic NLoS-only.

Shannon rate with bandwidth B_i, transmit power P_i, noise N_0:

Ri=Bilog21+Pigi,𝐪N0.	(12)

Transmission delay:

Titx=DitxRi.	(13)

D. Computation and Queueing Delay

For offload target o ∈ local∪M∪K, allocated CPU frequency f_i^alloc:

Ticomp={Cifialloc,o=local,[8pt]Ciremotefoalloc,o∈ℳ∪𝒦.	(14)

Queueing delay T_i^queue accumulates from per-server FIFO queues under resource contention. When multiple terminals share UAV m, bandwidth and CPU requests are normalized via proportional sharing with a minimum floor share.

E. End-to-End Delay and SLA Indicator

Total E2E delay:

Tie2e=Tisem+Titx+Tiqueue+Ticomp.	(15)

SLA satisfaction indicator for tier κ:

hiti=𝕀Tie2e≤Tκmax·𝕀S(μi)≥Sκmin.	(16)

Dense SLA quality (used in reward shaping):

qi=wdexp(−ρdTie2e)+wsS(μi)−ϕi,	(17)

ϕi=minϕmax, ρd2max(0,Tie2e−Tκmax)+ρs2max(0,Sκmin−S(μi)).	(18)

F. Energy Model

Local computation energy scales with allocated CPU; transmission energy with power and airtime; UAV propulsion dominates long-timescale energy:

Emprop(t)=ζ‖𝐯m(t)‖2Δ,	(19)

where v_m is UAV velocity from long-timescale action and ζ absorbs propulsion coefficient. Frame-level propulsion E_t^prop = Σ_m E_m^prop(t) is penalized in long-scale reward.

G. Two-Timescale Decision Structure

• Short action a_k^s: per-terminal offload encoding, μ_i, bandwidth request, CPU request;

• Long action a_t^l: UAV 3D displacement per frame (DDPG output in [-1,1] scaled by v_max).

IV. PROBLEM FORMULATION

A. Per-Tier Constrained CMDP

Define short-scale state s_k (global queue/energy loads, terminal local observations) and long-scale frame state s_t^f (aggregated SLA rates, UAV positions, tier weights).

Unconstrained objective: maximize expected discounted sum of SLA quality minus energy.

Per-tier SLA constraints: for each κ ∈ K_SLA,

lim infT→∞1Tt=1T𝔼[ρκ(t)]≥ηκ,	(20)

where ρ_κ^(t) is the empirical tier-κ SLA rate in frame t and η_κ is the target (e.g., η_critical=0.70, η_interactive=0.85).

Lagrangian short-slot reward (standard CMDP form—bounded violation cost):

rk=qk−ωeEk−κ∈𝒦SLAλκ1−hiti,k(κ),	(21)

where hit_i,k^(κ) is 1 only if the served task belongs to tier κ and satisfies (16); otherwise the indicator for non-matching tiers is treated as vacuous in implementation via tier tag of task i.

Dual ascent after each training episode (or frame window):

λκ←clipλκ+ηλ(ηκ−ρκ), [λmin,λmax].	(22)

Remark 1 (Stability). Unlike λ · q_k scaling, (21) keeps q_k unscaled so PPO advantage estimates remain bounded when λ_κ grows—critical for training stability (cf. v3 collapse with λ · q_k).

Remark 2 (vs. single λ). Single-λ CMDP enforces weighted average SLA only; per-tier deficits can hide in aggregate (Table II: single-λ ablation retains high overall SLA but tier bias persists in greedy baselines).

B. Model-Informed Feasibility Region

Before executing parsed short action a_k^s, the environment projects it to a_k^s ∈ F_k.

Step 1 — Similarity clamp:

μi←maxμi, μκ(i)sim.	(23)

Step 2 — Candidate offload set O_i = local∪M∪K (HAP disabled flag optionally removes K).

Step 3 — Deterministic delay estimate for candidate o using expected gain g (11), nominal (D_i,C_i), and resource estimates B_i,f_o from action:

Tie2e(o,μi)=Tisem+Titx(o)+Ticomp(o).	(24)

Step 4 — Delay-feasible set:

𝒪ifeas=o∈𝒪i:Tie2e(o,μi)≤α Tκ(i)max,	(25)

with delay margin α ∈ (0,1] (default 0.8) reserving headroom for task-size jitter (± 20\%) and NLoS samples.

Step 5 — Target selection: Map offload preference u_i ∈ [0,1] to index j = ⌊ u_i · |O_i^feas| ⌋ and select the j-th feasible target. If O_i^feas=∅, select argmin_o T_i^e2e(o,μ_i).

Property 1. F is a system-level action interface: any policy (DRL or heuristic) operates on the same feasible set, ensuring fair comparison under masked baselines (Section VII-A).

Property 2 (Decoupling). If O_i^feas is non-empty and dominated by HAP routes independent of UAV position, SLA satisfaction becomes less sensitive to q_m at moderate load—explaining why the single-timescale ablation remains competitive (90.7% SLA at 4.0 kJ, Table IV).

V. T2SEM-DRL ALGORITHM

A. Overview

T2Sem-DRL (Fig. 1) orchestrates three modules: (M1) short-timescale shared PPO; (M2) long-timescale DDPG; (M3) per-tier dual updater. Feasibility projection F is applied inside the environment step.

B. Short-Timescale Shared PPO (M1)

Motivation. Terminals share similar local structure (queue, tier one-hot, distances); parameter sharing improves sample efficiency.

Observation. Global state s_k for critic; local batch o_k,i per terminal for actor.

Action. Gaussian policy π_θ(a_k,i|o_k,i) with learnable log-std bounds [σ_min,σ_max] and entropy bonus β_ent.

PPO update with clip ε, GAE(λ) advantages, N_epoch epochs, gradient clip g_max:

LCLIP(θ)=𝔼minrt(θ)At, clip(rt(θ),1−ϵ,1+ϵ)At,	(26)

L(θ)=−LCLIP+cvLVF−βentH[πθ].	(27)

C. Long-Timescale DDPG (M2)

Frame state s_t^f includes UAV positions, loads, recent mean reward, SLA rates.

Actor μ_φ(s_t^f)arrow a_t^l; critic Q_ψ with target networks (φ',ψ').

Regularization (energy–SLA decoupling exploitation):

Lactor=−𝔼[Qψ(𝐬f,μϕ(𝐬f))]+βL2‖μϕ(𝐬f)‖22+βsm‖𝐚tl−𝐚t−1l‖22.	(28)

Frame reward:

Rt=1Ksk∈frame trk−ωtrajEtprop.	(29)

D. Filtered Behavioral Cloning Warm-Start

Greedy demonstrations without mask achieve 0% SLA; filtered BC collects per-terminal (o,a) pairs only from episodes with hit=1, mixing random and greedy explorers. Pretrain actor with MSE loss for E_BC epochs before RL.

E. Training Procedure

Algorithm 1 T2Sem-DRL v4 Training 1: Collect filtered demonstrations D; BC-pretrain π_θ 2: Initialize λ_κ arrow λ_init for all tiers 3: for frame t=1 to T_max do 4: a_t^l arrow DDPG(s_t^f); apply UAV motion 5: for slot k=1 to K_s do 6: Sample a_k^s ~ π_θ(·|o_k) 7: a_k^s arrow Π_F(a_k^s) 8: Execute; observe r_k from (21) 9: end for 10: PPO update; DDPG update; dual ascent (22) 11: end for

VI. IMPLEMENTATION DETAILS

| Hyperparameter | Value |
| --- | --- |
| Users / terminals / UAVs / HAPs | 8 / 16 / 3 / 2 |
| Area L | 500 m |
| Slots per frame K_s | 10 |
| Frames per episode | 200 |
| Task arrival rate | 0.25 / terminal / slot |
| D_i, C_i | 2 Mb, 1 G cycles |
| PPO hidden dim | 256 |
| PPO lr, clip, epochs | 3×10^-4, 0.2, 10 |
| Entropy coef. | 0.01 |
| DDPG actor/critic lr | 10^-4 / 10^-3 |
| BC demos / epochs | 30 / 150 |
| Training episodes | 300 |
| Eval episodes × seeds | 30 × 42,123,456 |
| α, δ_s, channel est. | 0.8, 0.05, expected |
| η_κ | (0.70, 0.85, 0.85) |
| η_λ, λ_max | 0.1, 8.0 |
| Device | CPU (default) |

Checkpoint selection uses periodic eval SLA (not noisy last-episode training metric).

VII. PERFORMANCE EVALUATION

A. Experimental Protocol

All methods—including Random, Greedy, TJCCT, Local-only—share the same feasibility mask as a system action interface (Property 1). Vanilla mask-off baselines appear in Appendix A only.

Training: improved_v4_pertier.yaml, 300 episodes, seeds 42,123,456. Evaluation: 30 deterministic episodes per seed.

B. Main Comparison (Table I)

TABLE I OVERALL PERFORMANCE (MASKED BASELINES). Mean over 3 seeds × 30 eval episodes. Bold: best per column. SLA↑, Delay↓. Energy reported separately in Table I-b to avoid mixing exploration-inflated trajectory cost with heuristic static policies.

| Method | SLA Rate (%) | Avg. Delay (s) |
| --- | --- | --- |
| Random | 78.47 ± 0.06 | 0.159 |
| Greedy | 87.77 ± 0.07 | 0.122 |
| TJCCT-Lite | 86.64 ± 0.07 | 0.266 |
| Local-only | 86.05 ± 0.37 | 0.209 |
| Independent PPO | 91.41 ± 0.25 | 0.145 |
| Single-timescale | 88.85 ± 0.43 | 0.145 |
| T2Sem-DRL | 93.99 ± 1.74 | 0.100 |

TJCCT-Lite uses K-means long-scale placement and priority water-filling short-scale allocation; it is no longer identical to greedy. Single-λ CMDP is reported in Table IV because its final evidence comes from the ablation protocol.

T2Sem-DRL improves SLA by +6.22 pp over masked greedy and by +7.35 pp over TJCCT-Lite. This is the secondary gain after feasibility projection and semantic modeling (Table V). We do not compare propulsion energy in this table; see Table I-b.

Fig. 2. Overall SLA comparison under the shared feasibility mask.

TABLE I-b ENERGY OPERATING POINTS (UNIFIED REPORTING). E_SLA = E_total / (SLA rate) (J per satisfied-task fraction; lower is better only when SLA is held constant).

| Operating point | Config | SLA (%) | Energy (J) | E_SLA (kJ) | Role |
| --- | --- | --- | --- | --- | --- |
| R1 Service default | Full two-timescale checkpoint | 93.99 | 7,856 | 8.4 | Best SLA in main protocol |
| R2 Pareto P4 | High w_traj, L2=0.12 | 91.1 | 16,181 | 17.8 | Historical Pareto operating point |
| R3 No DDPG | Single-timescale | 88.85 | 4,065 | 4.6 | Low-energy learned policy |
| Masked greedy | Heuristic + F | 87.77 | 3,630 | 4.1 | Static baseline |

At R1, the full model buys +6.22 pp SLA over greedy at higher energy. At R3, removing DDPG nearly matches greedy energy while retaining a small SLA gain. We therefore present energy as an operating-point trade-off, not as a universal dominance claim.

Fig. 3. Delay and energy comparison across representative methods.

C. Per-Tier SLA (Table II, Fig. 4)

TABLE II PER-TIER SLA SATISFACTION (%).

| Method | Critical | Interactive | Best-effort | Overall |
| --- | --- | --- | --- | --- |
| Random | 42.9 | 91.5 | 98.5 | 77.6 |
| Greedy/TJCCT* | 91.6 | 49.9 | 100.0 | 80.5 |
| Single-λ CMDP† | 83.8 | 94.3 | 100.0 | 92.8 |
| Local-only | 56.7 | 99.9 | 98.2 | 85.0 |
| T2Sem-DRL† | 76.0 | 99.2 | 100.0 | 92.2 |

\*Per-tier baseline rows are from a separate masked per-tier diagnostic run; their overall column therefore differs from Table I, which uses the main 3-seed × 30-episode protocol. †Final v4.3 ablation-scale eval (3 seeds × 300 episodes).

The final full model strongly protects interactive and best-effort tiers, while critical-tier SLA remains the hardest target. Greedy sacrifices interactive QoS (49.9%) despite high critical SLA—motivating explicit per-tier reporting even when aggregate SLA is high.

Fig. 4. Per-tier SLA satisfaction for critical, interactive, and best-effort tasks.

D. Energy–SLA Pareto (Table III, Fig. 5)

TABLE III ENERGY–SLA PARETO (2 SEEDS × 150 TRAIN EP).

| Point | w_traj | L2 coef. | SLA (%) | Energy (J) |
| --- | --- | --- | --- | --- |
| P1 min pressure | 0.05 | 0.00 | 87.9 | 687,653 |
| P2 low | 0.25 | 0.01 | 85.0 | 51,771 |
| P3 v4 default | 0.60 | 0.05 | 90.6 | 21,094 |
| P4 high pressure | 1.20 | 0.12 | 91.1 | 16,181 |
| Masked-random | — | — | 77.8 | 3,975 |

Stronger trajectory regularization sharply reduces propulsion energy while maintaining high SLA after the low-pressure regime; the P2 dip shows that the relation is not a universal monotonic law but an operating-point trade-off once F handles short-timescale routing.

Fig. 5. Energy-SLA Pareto operating points and masked baselines.

E. Ablation Study (Table IV, Fig. 6)

TABLE IV COMPONENT ABLATION (3 SEEDS × 300 TRAIN EP). †Upper bound without contention.

| Variant | SLA (%) | Energy (J) | Critical | Interactive |
| --- | --- | --- | --- | --- |
| Full | 92.17 | 15,366 | 76.0 | 99.2 |
| w/o mask | 60.39 | 61,104 | 5.2 | 78.5 |
| w/o per-tier λ | 92.80 | 17,518 | 83.8 | 94.3 |
| w/o CMDP | 92.04 | 16,418 | 78.0 | 99.3 |
| Pure DRL | 59.91 | 57,712 | 2.6 | 70.8 |
| w/o semantic | 46.17 | 43,776 | 1.8 | 36.5 |
| Single-timescale | 90.70 | 4,013 | 73.8 | 98.3 |
| Fixed μ | 65.72 | 45,852 | 0.0 | 99.0 |
| w/o SLA penalty | 92.18 | 16,543 | 86.0 | 90.7 |
| w/o contention† | 100.00 | 56,026 | 100.0 | 100.0 |

Findings (honest hierarchy): semantic +46.0 pp > mask +31.8 pp > RL over masked greedy +6.2 pp. Per-tier λ_κ and SLA-penalty variants mainly change tier allocation rather than aggregate SLA. Single-timescale loses 1.47 pp SLA in the ablation protocol but uses 3.8× less energy.

Fig. 6. Ablation study showing the gap between each variant and the full model.

F. Cumulative Component Build-up (Table V)

TABLE V COMPONENT CONTRIBUTION DECOMPOSITION. Forward path from heuristic to learned policy; ablation rows isolate major system levers.

| Stage | Configuration | SLA (%) | Interpretation |
| --- | --- | --- | --- |
| S0 | Pure DRL (no F, no semantic model) | 59.9 | mask-free lower reference |
| S1 | Masked greedy (F + heuristic) | 87.8 | interface gain over S0 |
| S2 | + Masked PPO (learned policy within F) | 94.0 | +6.2 pp over S1 |
| S3 | + Per-tier λ_κ (ablation full model) | 92.2 | diagnostic |
| — | Remove F from S3 (ablation) | 60.4 | −31.8 |
| — | Remove semantic model from S3 (ablation) | 46.2 | −46.0 |

S0→S1 mixes policy and interface changes, so the negative ablation rows isolate mask and semantic levers from S3 rather than claiming a strictly monotonic build-up. Narrative: model-informed F and semantics dominate; RL refines within the feasible shell, while per-tier duals are best interpreted through tier allocation rather than aggregate SLA alone.

G. Mask Sensitivity (Fig. 7)

Test-time variation without retraining: SLA peaks at α=0.6–0.7 (93–94%); default α=0.8 yields 87.4%; optimistic α=1.0 drops to 83.0%. Expected-channel estimate matches realistic LoS mixture; similarity margin trades robustness vs. compression cost.

Fig. 7. Mask sensitivity under delay margin, channel estimate, and similarity margin changes.

H. Discussion, Limitations, and DDPG Extension

Positioning. Primary scientific contribution is the model-informed per-tier semantic SLA interface (F + semantic dynamics + tier diagnostics), not a new two-timescale DRL architecture. Masked PPO adds +6.2 pp over masked greedy; two-timescale DDPG is an optional SLA-energy operating mode.

Masked baseline protocol is essential: removing the mask drops full-model SLA from 92.17% to 60.39%. Energy: main Table I omits energy; Table I-b reports operating points and explicitly avoids claiming universal energy superiority over static heuristics.

CMDP theory vs. finite horizon. Per-tier dual ascent targets long-run constraint satisfaction; 300-episode training is a finite proxy, so Fig. 8 reports empirical λ_κ trajectories and violation traces rather than claiming asymptotic constraint satisfaction.

Fig. 8. Per-tier dual variables and violation traces during training.

DDPG extension (optional). Two-timescale DDPG is retained as a controllable operating mode. Under the mobility-critical stress test (Table VI)—a low-HAP-capacity, high-density configuration where feasible routing depends heavily on UAV placement—both policies degrade sharply, but the two-timescale full model retains a meaningful margin over the single-timescale variant (16.4% vs. 1.7% overall SLA). The large seed variance (±13.6 pp) and poor absolute SLA indicate that this regime is reported as a limitation rather than proof that trajectory learning alone resolves hard coverage failures.

TABLE VI MOBILITY-CRITICAL STRESS TEST (3 SEEDS). Low HAP capacity with position-dependent feasibility; both policies are far from their default-load SLA (Table I).

| Policy | Overall SLA (%) | Avg. Delay (s) | Energy (J) |
| --- | --- | --- | --- |
| Two-timescale (full) | 16.4 ± 13.6 | 0.870 | 53,959 |
| Single-timescale | 1.7 ± 2.9 | 2.403 | 8,509 |

Two-timescale UAV trajectory learning recovers a small but non-trivial SLA fraction that the static single-timescale policy cannot, motivating its retention as an optional operating mode for coverage-limited deployments.

Future work: semantic MAPPO baseline [3], independent TJCCT implementation, statistical tests across seeds, hardware-in-the-loop.

VIII. CONCLUSION

This paper reframes hierarchical UAV–HAP semantic MEC offloading around a model-informed per-tier SLA interface rather than a monolithic DRL framework. Empirically, masked greedy reaches 87.77% overall SLA and TJCCT-Lite reaches 86.64%, while pure DRL without model guidance reaches only 59.91%.

The proposed interface comprises: (1) feasibility projection F (+31.8 pp when restored); (2) semantic compression and remote-load modeling (+46.0 pp); (3) per-tier CMDP with λ_κ for tier-level diagnostics; (4) optional masked PPO (+6.2 pp over masked greedy). Honest hierarchy: semantic > mask > RL, while per-tier λ_κ primarily changes tier allocation.

Main-protocol evaluation achieves 93.99±1.74% overall SLA and 0.100 s mean delay. Energy should be interpreted as an operating-point trade-off: the full model uses 7.9 kJ, while the single-timescale variant uses 4.1 kJ at 88.85% SLA. The mobility-critical stress test remains a limitation rather than proof that trajectory learning alone resolves hard coverage failures.

Limitations include critical-tier fragility in some ablations, seed-dependent energy variance, and poor absolute SLA in the mobility-critical stress test (Table VI).

REFERENCES

[1] H. Hao et al., "Joint Task Offloading, Resource Allocation, and Trajectory Design for Multi-UAV Cooperative Edge Computing With Task Priority," IEEE Trans. Mobile Comput., vol. 23, no. 9, pp. 8649–8665, Sep. 2024.

[2] L. Liu et al., "DRL-Based Trajectory Optimization and Task Offloading in Hierarchical Aerial MEC," IEEE Open J. Commun. Soc., vol. 6, pp. 1234–1248, 2025.

[3] Z. Wang et al., "Online Multi-Task Offloading for Semantic-Aware Edge Computing Systems," IEEE Trans. Cogn. Commun. Netw., early access, 2024.

[4] L. Chen et al., "Computation Offloading and Resource Allocation Strategy Considering User Mobility in Multi-UAV Assisted Semantic Communication Networks," Electronics, vol. 14, no. 20, 2025.

[5] W. Zhang et al., "TJCCT: A Two-timescale Approach for UAV-assisted Mobile Edge Computing," IEEE Trans. Veh. Technol., early access, 2024.

[6] S. Zhang, L. Liu, and X. Chen, "Joint Service Caching and Resource Allocation in Collaborative MEC: A DRL-Based Two-timescale Approach," IEEE Trans. Netw. Sci. Eng., vol. 11, no. 2, pp. 1456–1470, Mar.–Apr. 2024.

[7] M. Zhou et al., "Two-Timescale Model Caching and Resource Allocation for Edge-Enabled AI-Generated Content Services," IEEE Trans. Mobile Comput., early access, 2025.

[8] H. Du et al., "High-Altitude Platform Station (HAPS) Assisted Mobile Edge Computing: Architecture and Task Offloading," IEEE Netw., vol. 36, no. 5, pp. 140–147, Sep.–Oct. 2022.

[9] D. Gündüz et al., "Beyond Transmitting Bits: Context, Semantics, and Task-Oriented Communications," IEEE J. Sel. Areas Commun., vol. 41, no. 1, pp. 5–41, Jan. 2023.

[10] W. Xu et al., "Semantic Communications: Principles and Challenges," arXiv:2201.01383, 2022.

[11] F. Song et al., "UAV-Enabled Semantic Communication in Mobile Edge Computing Under Jamming Attacks," IEEE Trans. Wireless Commun., vol. 23, no. 12, pp. 19876–19890, Dec. 2024.

[12] Z. Shao et al., "Deep Reinforcement Learning-Based Resource Management for UAV-Assisted Mobile Edge Computing Against Jamming," IEEE Trans. Mobile Comput., vol. 23, no. 12, pp. 13358–13374, Dec. 2024.

[13] G. Wu et al., "Joint Task Offloading and Resource Allocation in Multi-UAV Multi-Server Systems," IEEE Trans. Veh. Technol., vol. 73, no. 8, pp. 11964–11978, Aug. 2024.

[14] S. Liu et al., "Learning-Based Resource Management Optimization for UAV-Assisted MEC Against Jamming," IEEE Trans. Commun., vol. 72, no. 8, pp. 4873–4886, Aug. 2024.

[15] C. Ren et al., "Joint Edge Computing and Semantic Communication in UAV-Enabled Networks," IEEE Commun. Lett., vol. 28, no. 12, pp. 2845–2849, Dec. 2024.

[16] M. Chen et al., "Distributed Learning for Wireless Communications," IEEE Commun. Surveys Tuts., vol. 24, no. 4, pp. 2035–2072, 4th Quart. 2022.

[17] B. Hu et al., "Computation Offloading and Resource Allocation in Mixed Cloud/Vehicular-Fog Computing Systems," IEEE Trans. Mobile Comput., vol. 24, no. 9, pp. 8612–8624, Sep. 2025.

[18] F. Liu et al., "Rate-Adaptable Multitask-Oriented Semantic Communication," IEEE Internet Things J., early access, 2024.

[19] C. Ren et al., "Multimodal Virtual Semantic Communication for Tiny-Machine-Learning-Based UAV Task Execution," IEEE Internet Things J., vol. 11, no. 19, pp. 30864–30874, Oct. 2024.

[20] H. Zhang et al., "DRL-Driven Dynamic Resource Allocation for Task-Oriented Semantic Communication," IEEE Trans. Commun., vol. 71, no. 7, pp. 3992–4004, Jul. 2023.

[21] W. Liu et al., "Unmanned Aerial Vehicle-Enabled Mobile Edge Computing for Semantic Communications," Comput. Commun., vol. 235, p. 108411, Jan. 2026.

[22] Y. Wang et al., "Task Offloading and Resource Allocation for Semantic Communication in Air-Ground MEC Networks," in Proc. IEEE VTC-Fall, 2025, pp. 1–7.

APPENDIX A: MASK-OFF BASELINES

| Method | Overall SLA (%) |
| --- | --- |
| Greedy (mask-off) | 0.0 |
| Random (mask-off) | 33.2 |
| Greedy (masked) | 87.8 |

APPENDIX B: CUMULATIVE COMPONENT BUILD-UP

| Stage | SLA (%) | Δ |
| --- | --- | --- |
| Pure DRL | 59.9 | — |
| + feasibility mask / heuristic shell | 87.8 | +27.9 |
| + masked PPO | 94.0 | +6.2 |
| Full ablation model | 92.2 | tier allocation diagnostic |

APPENDIX C: REPRODUCIBILITY

cd t2sem-mec-v2
python experiments/run_all.py --config configs/improved_v4_pertier.yaml --variants A1_full --device cpu
python scripts/per_tier_eval.py --config configs/improved_v4_pertier.yaml
cd ../T2Sem-DRL-v4-paper && python scripts/plot_paper_figures.py